Give Your Kitchen a New Look

If you’re looking for a way to transform an outdated kitchen, a fresh coat of cabinet paint can go a long way without breaking the bank. To make sure your cabinet painting project goes as smoothly as possible, the experts at General Paint have compiled a step-by-step guide.

How to Paint Kitchen Cupboards

#1 Clear the Area

First, empty the cabinets, clear off all counters and remove all freestanding appliances. You should also relocate other furniture such as a kitchen table and chairs to another room. Then remove all hinges, hardware, doors and drawers from your cabinets and label each door with a numbered piece of masking tape, so you can remember which one goes where. Finally, cover the floors with drop cloths, laying each cloth as close to the baseboard as possible.

#2 The Caution Tape

Next, apply 1-inch or 2-inch FrogTape® along the edges of the cabinets, as well as any window frames that you will not be painting.

#3 Lather, Rinse, Repeat...

Start prepping your cabinet boxes and doors by washing down the surfaces with warm water and a good household detergent. This will remove soot, grease, smoke and airborne dirt. Next, use 100-grit sandpaper to make all of the surfaces smooth. To achieve the most professional-style results, prime the cabinet boxes and doors using a Baseline Primer and an angled 1-inch or 2-inch brush. Starting at the top of the cabinet, brush on primer across the grain of the cabinet by holding the brush near the base of the handle. This offers good control, so it is well suited for detail work such as getting close to edges or molding.

#4  Good Practices

Once your primer has dried, pour a fair amount of Latex Paint into the roller pan and use an angled brush and a mini foam roller to apply your paint. Similar to the primer, start at the top of the cabinet and brush across the grain. Apply two coats to each cabinet, lightly sanding with 100-grit sandpaper in-between coats. When all cabinet boxes and doors have been painted, let dry for 48 hours.

#5 Reassemble and Redecorate

When the final coat has dried remove the tape from each door and reinstall the doors based on their original number. You can then redecorate your new kitchen by installing new cabinet hardware and reassembling your kitchen.

Canadian Home Trends Magazine gives you a personal tour of the most stunning homes and condos. In each issue, you are given the tools to recreate designer spaces you’ve always dreamt of having at home, in-depth renovation and design advice, and the best places to shop. To subscribe, visit www.CanadianHomeTrends.com